STM32 Firmware — Study Notes

Servo PWM  |  UART + DMA  |  I2C LCD Display

PART A — Servo + UART / DMA

🗺️  What the firmware does

Four things running concurrently:

- Pot on PA4 → ADC reads continuously → maps to servo angle

- TIM3 outputs 50 Hz PWM → drives servo motor

- DMA quietly watches USART2 RX — fires a callback each time a byte arrives

- Every loop appends an ADC reading to a buffer. Press J → entire backlog sent over serial in one shot

💡 Key idea: DMA handles UART in the background — the CPU never sits waiting for bytes. The J-key callback just sets a flag; the main loop checks it and fires the transmit.

📖  Quick vocab

| Word | What it means |
| --- | --- |
| HAL | Hardware Abstraction Layer — ST's library so you don't write raw register code |
| Peripheral | A hardware block inside the chip (ADC, timer, GPIO, USART, I2C...) |
| PSC / Prescaler | Divides the clock before it reaches the timer counter |
| ARR | Auto-Reload Register — counter resets when it hits this value |
| CCR | Compare Register — where in the count the PWM output flips |
| PWM | Pulse-Width Modulation — fast toggling to control power / servo angle |
| IRQ / Interrupt | Hardware signal that pauses main code and runs a handler function |
| DMA | Direct Memory Access — peripheral copies data straight to RAM, no CPU needed |
| UART / USART | Serial comms protocol — TX sends, RX receives, one byte at a time |
| Baud rate | Bits per second. 9600 baud = ~960 bytes/sec (8 data + 1 stop bit) |
| volatile | Tells the compiler: don't optimise this away — hardware may change it |
| I2C | Two-wire bus (SDA + SCL) for talking to peripheral chips like LCD expanders |
| I2C address | 7-bit number identifying a device on the bus — shift left 1 for HAL |

⏱️  SystemClock_Config()

HSI oscillator at 16 MHz, no PLL, all bus dividers = 1.

SYSCLK = HCLK = PCLK1 = 16 MHz  ← all timer and UART maths uses this

🚀  MX_DMA_Init() — setting up DMA

Must be called before the UART that uses it. CubeMX always puts MX_DMA_Init() first.

- __HAL_RCC_DMA1_CLK_ENABLE() — turns on the DMA1 clock (off by default)

- Configures NVIC priority for DMA1 channels 1 and 2/3 — these carry USART2 RX and TX traffic

⚠️ Order matters: If MX_UART_Init runs before MX_DMA_Init, HAL tries to link UART to a DMA that hasn't been set up yet and silently breaks.

What DMA does for UART

- You say: watch USART2's data register, copy each byte to this RAM address

- DMA does that with zero CPU cycles per byte

- When N bytes arrive, DMA fires a completion interrupt → HAL calls your callback

- You call HAL_UART_Receive_DMA again in the callback to keep listening

📡  MX_USART2_UART_Init() — serial comms

Configures USART2 as the virtual COM port — shows up when you plug the Nucleo into a PC via USB.

| Setting | Value | Why |
| --- | --- | --- |
| BaudRate | 9600 | Matches your terminal app — both ends must agree |
| WordLength | 8 bits | Standard byte size |
| StopBits | 1 | Standard framing |
| Parity | NONE | No error-checking bit |
| Mode | TX_RX | Both sending and receiving enabled |
| OverSampling | 16x | Better noise rejection at low baud rates |
| FIFO mode | DISABLED | Bytes handled one at a time — keeps callback logic simple |

💡 9600 baud is slow by design. At 500 ms per loop iteration you're only accumulating ~2 lines/second. 9600 is plenty — and slower speeds are more tolerant of cable noise.

🎛️  MX_ADC1_Init() — reading the pot

Reads pin PA4 (channel 4). Converts 0–3.3V to 0–4095. This version uses ContinuousConvMode = ENABLE — the ADC keeps converting on its own, you just poll for the latest result.

✅ vs previous version: Before: ContinuousConvMode OFF — call HAL_ADC_Start() every loop. Now: call once before the loop, then just poll. The ADC is always running.

Mapping to servo angle

servo_pos = 10 + (adcValue * 10) / 4095;

// 0    -> CCR=10 -> 1 ms pulse -> 0 degrees

// 4095 -> CCR=20 -> 2 ms pulse -> 180 degrees

🔄  MX_TIM3_Init() — servo PWM

This version uses ARR = 65535 (the 16-bit max). The PWM frequency is now ~0.15 Hz — much slower than the ideal 50 Hz for servos.

16,000,000 / 1600 / 65536 = ~0.15 Hz   <- not ideal

16,000,000 / 1600 / 200   = 50 Hz       <- what you want for servos

⚠️ ARR=65535: This is the CubeMX default if you forget to set the period. The servo will still move when CCR changes but won't get proper position updates 50 times/second. Change ARR to 199 for correct operation.

🔁  The while(1) loop

HAL_ADC_Start(&hadc1);                       // once — continuous mode

HAL_UART_Receive_DMA(&huart2, rxBuffer, 1);  // arm DMA RX

while (1) {

// 1. Update servo

if (HAL_ADC_PollForConversion(&hadc1, 10) == HAL_OK) {

adcValue = HAL_ADC_GetValue(&hadc1);

servo_pos = 10 + (adcValue * 10) / 4095;

__HAL_TIM_SET_COMPARE(&htim3, TIM_CHANNEL_1, servo_pos);

}

// 2. Append reading to buffer (if space remains)

if (bufferLength < 400) {

bufferLength += sprintf(&txBuffer[bufferLength],

"%lu. ADC: %lu\r\n", messageCount++, adcValue);

}

// 3. If J pressed, dump the whole backlog

if (jKeyPressed == 1) {

HAL_UART_Transmit_DMA(&huart2, txBuffer, bufferLength);

bufferLength = 0;    // reset write cursor

jKeyPressed = 0;     // clear flag

}

// 4. Blink LED and pace the loop

HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5);

HAL_Delay(500);          // ~1 reading appended per second

}

The sprintf buffer trick

txBuffer is a 512-byte array. bufferLength is a cursor — it tracks where to write next. Each sprintf() writes at &txBuffer[bufferLength] and returns the number of chars written, which gets added to bufferLength. When J is pressed, send the whole array up to bufferLength in one DMA transfer.

⚠️ Safety check: The if (bufferLength < 400) guard stops at 400 of 512 bytes, leaving room so sprintf can't overflow into memory past the buffer. Without it, a long run would silently corrupt whatever's stored after txBuffer.

The J-key callback

void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart) {

if (huart->Instance == USART2) {

if (rxBuffer[0] == 'J' || rxBuffer[0] == 'j') {

jKeyPressed = 1;   // set flag only — don't TX here

}

// Re-arm DMA to catch the next byte

HAL_UART_Receive_DMA(&huart2, rxBuffer, 1);

}

}

🚫 Don't transmit inside a callback. Callbacks run in interrupt context. HAL_UART_Transmit_DMA here while the main loop might also be touching UART can cause conflicts. Set a flag, handle it in main.

🧮  Timer formula cheat sheet

Tick freq  = PCLK1 / (PSC + 1)

Period     = (ARR + 1) / tick_freq    <- seconds

PWM freq   = 1 / period

Duty cycle = CCR / (ARR + 1)

TIM3 (this code): 16e6 / 1600 = 10 kHz  |  65536 ticks = 6.5 s per cycle

TIM3 (correct):   16e6 / 1600 = 10 kHz  |    200 ticks = 20 ms = 50 Hz

🪲  Common gotchas (UART / DMA)

| Gotcha | Fix |
| --- | --- |
| MX_UART_Init before MX_DMA_Init | DMA must be inited first — swap the order |
| HAL_UART_Receive_DMA called once, never re-armed | Call it again inside HAL_UART_RxCpltCallback |
| Transmitting inside interrupt callback | Set a flag only — TX in main loop |
| bufferLength not reset after send | Set to 0 after HAL_UART_Transmit_DMA |
| ARR=65535 for servo | Change to 199 for correct 50 Hz PWM |
| volatile missing on jKeyPressed | Without it the compiler may cache the value and never see the IRQ update |
| GPIO clock not enabled | Add __HAL_RCC_GPIOx_CLK_ENABLE() |

PART B — I2C LCD Display

🖥️  What the LCD code does

Drives a 16x2 character LCD (HD44780 controller) through a PCF8574 I2C expander chip. Instead of needing 6-8 GPIO wires to the LCD, you only need 2: SDA and SCL.

The hardware chain

| Step | What happens |
| --- | --- |
| 1 | MCU sends bytes over I2C to address 0x27 (the PCF8574 expander) |
| 2 | PCF8574 converts those bytes to 8 GPIO output pins |
| 3 | Those 8 pins connect to the HD44780 LCD (4 data + RS + E + RW + backlight) |
| 4 | HD44780 interprets the signals and updates what's on the screen |

💡 Why I2C? Without the PCF8574 expander you'd need 6+ MCU pins wired directly to the LCD. I2C gives you the same control over just 2 pins.

📬  I2C and the device address

I2C is a two-wire bus — SDA (data) and SCL (clock). Multiple devices share the same wires, each identified by a 7-bit address.

Address shifting — why 0x27 << 1

#define LCD_I2C_ADDR  0x27 << 1   // = 0x4E

// HAL's I2C functions expect an 8-bit value:

//   bits [7:1] = 7-bit device address

//   bit  [0]   = R/W (HAL fills this in — 0=write, 1=read)

//

// So shift the 7-bit address left by 1 to make room.

⚠️ Common mistake: Passing 0x27 (unshifted) to HAL_I2C_Master_Transmit. HAL reads it as address 0x13 and gets no response — looks like the LCD is dead.

🔢  4-bit mode — why only 4 data pins?

The HD44780 supports 8-bit mode (8 data pins) or 4-bit mode (4 data pins). The PCF8574 only has 8 GPIO pins total — some are used for RS, E, RW, and backlight — so 4 data pins is all that's left. 4-bit mode it is.

In 4-bit mode, each byte is sent in two halves (nibbles):

- Send upper nibble (bits 7-4) first

- Then lower nibble (bits 3-0)

- Each nibble needs an Enable (E) pulse — a HIGH then LOW on the E pin — to tell the LCD to latch it

🗂️  PCF8574 pin mapping

Each bit in the byte you send to the PCF8574 controls one LCD pin:

| Bit | Pin | Purpose |
| --- | --- | --- |
| 7 | D7 | LCD data bit 7 (MSB of upper nibble) |
| 6 | D6 | LCD data bit 6 |
| 5 | D5 | LCD data bit 5 |
| 4 | D4 | LCD data bit 4 (LSB of upper nibble) |
| 3 | BL | Backlight — always 1 (0x08) to keep screen lit |
| 2 | E | Enable — pulse HIGH then LOW to latch data into LCD |
| 1 | RW | Read/Write — always 0 (write only) |
| 0 | RS | Register Select — 0 = command, 1 = character data |

⚙️  lcd_send_internal() — the core function

This is the engine of the whole driver. Everything else calls it. It takes one byte and sends it as two nibbles, each with an E pulse, totalling 4 bytes sent over I2C per call.

// Fixed signature (original has a bug — see below)

void lcd_send_internal(uint8_t data, uint8_t rs) {

tx_data[0] = (data >> 4) | 0x08 | rs;   // upper nibble, E=0

tx_data[1] = tx_data[0] | 0x04;          // upper nibble, E=1 (pulse)

tx_data[2] = (data & 0x0F) | 0x08 | rs;  // lower nibble, E=0

tx_data[3] = tx_data[2] | 0x04;          // lower nibble, E=1 (pulse)

HAL_I2C_Master_Transmit(&hi2c1, LCD_I2C_ADDR, tx_data, 4, HAL_MAX_DELAY);

HAL_Delay(2);

}

Breaking down each byte

| Expression | What it does |
| --- | --- |
| data >> 4 | Shift top 4 bits down to [3:0] — these go to LCD pins D4-D7 |
| 0x08 | Force bit 3 = 1 — keeps the backlight ON |
| rs | Put RS in bit 0: 0 = command, 1 = character data |
| ¦ 0x04 | Force bit 2 = 1 — the E (Enable) pulse, latches data into LCD |
| data & 0x0F | Mask the lower 4 bits — second nibble |

💡 Why 4 bytes per character? Byte 0: upper nibble, E low. Byte 1: upper nibble, E high (rising edge latches). Byte 2: lower nibble, E low. Byte 3: lower nibble, E high (latches again). The LCD captures data on the E rising edge.

🚫 Bug in the original code: lcd_send_internal(uint8_t rs) only takes rs as a parameter but uses a variable called 'data' that isn't passed in — won't compile. Fix: change signature to lcd_send_internal(uint8_t data, uint8_t rs) and update the callers.

🔌  lcd_init() — startup sequence

The HD44780 needs a specific init sequence after power-on. It boots in an unknown state — you can't just start sending commands.

HAL_Delay(50);           // wait for LCD power rail to stabilise

// Datasheet reset sequence: send 0x03 three times, then 0x02

// This reliably gets the LCD into 4-bit mode from any start state

uint8_t init_cmds[] = {0x03, 0x03, 0x03, 0x02};

for (int i = 0; i < 4; i++) { ... send each ... }

lcd_send_cmd(0x28);      // 4-bit mode, 2 lines, 5x8 font

lcd_send_cmd(0x08);      // display OFF

lcd_send_cmd(0x01);      // clear display

HAL_Delay(2);            // clear takes ~1.5 ms

lcd_send_cmd(0x06);      // entry mode: cursor moves right, no scroll

lcd_send_cmd(0x0C);      // display ON, no cursor, no blink

HD44780 command reference

| Command | Hex | What it does |
| --- | --- | --- |
| Function Set | 0x28 | 4-bit, 2 lines, 5x8 dots |
| Display off | 0x08 | Screen blank — used mid-init |
| Clear | 0x01 | Wipe screen, cursor goes to (0,0) — takes ~1.5 ms |
| Entry mode | 0x06 | Cursor moves right after each char, display doesn't shift |
| Display on | 0x0C | Screen on, cursor invisible, no blink |
| Set cursor row 0 | 0x80 + col | Row 0 DDRAM starts at 0x80 |
| Set cursor row 1 | 0xC0 + col | Row 1 DDRAM starts at 0xC0 |

📍  lcd_set_cursor() — moving around

void lcd_set_cursor(uint8_t row, uint8_t col) {

uint8_t addr = (row == 0) ? 0x80 + col : 0xC0 + col;

lcd_send_cmd(addr);

}

// Examples:

// lcd_set_cursor(0, 0)  -> 0x80  (top-left)

// lcd_set_cursor(1, 0)  -> 0xC0  (bottom-left)

// lcd_set_cursor(1, 11) -> 0xCB  (row 1, 12th character)

The 0x80 and 0xC0 offsets come from the HD44780 DDRAM memory map. Row 0 is at address 0x00 and row 1 at 0x40. The 'Set DDRAM Address' command has bit 7 = 1, so: 0x80 + 0x00 = row 0, 0x80 + 0x40 = 0xC0 = row 1.

📟  Putting it together in main()

lcd_init();

lcd_clear();

lcd_set_cursor(0, 0);

lcd_puts("S I D 2 5 8 9 5 2 7 3");    // row 0

lcd_set_cursor(1, 0);

lcd_puts("M E C H A T R O N I C S");   // row 1

lcd_set_cursor(1, 11);

lcd_puts("1");                           // overwrite position (1,11)

💡 Spaced letters: Writing 'S I D' with spaces between each character is just aesthetic — spreads them across more of the 16-character display width.

lcd_puts() — writing strings

void lcd_puts(char* str) {

while (*str) {              // loop until null terminator (\0)

lcd_send_data(*str++);  // send char, advance pointer

}

}

Classic C string walk. *str++ dereferences the pointer to get the character, sends it to the LCD, then increments the pointer. Stops when it hits the \0 at the end of the string.

🪲  LCD / I2C gotchas

| Gotcha | Fix |
| --- | --- |
| Address not shifted — 0x27 instead of 0x27<<1 | HAL needs 8-bit address — always left-shift by 1 |
| No HAL_Delay(50) at start of lcd_init | LCD needs 50 ms after power-on before any commands |
| Missing HAL_Delay(2) after clear command | Clear takes ~1.5 ms — skip this and the next command gets ignored |
| lcd_send_internal bug — data not passed as parameter | Add data as first parameter and update callers |
| I2C pins not open-drain | I2C must be open-drain — CubeMX sets this automatically for I2C AF pins |
| Backlight bit (0x08) missing from byte | Screen works but is dark — easy to mistake for broken hardware |
| Wrong I2C address — expander module may use 0x3F | Check your PCF8574 variant — A4 address has 0x3F, not 0x27 |